Identification_Information:

Citation:

Citation_Information:

Originator: Grantz, A.

Originator: Scott, R.A.

Originator: Drachev, S.S.

Originator: Moore, T.E.

Originator: Howard, J.P.

Publication_Date: 2010

Title:

Map Showing the Sedimentary Successions of the Arctic Region (58°-64° to 90° N) that may be prospective for Hydrocarbons

Edition: 1.0

Geospatial_Data_Presentation_Form: vector digital data

Online_Linkage:

Description:

Abstract:

This digital map database, interpreted from previously published and unpublished geological and geophysical maps, and new work by the authors, presents the distribution, tectono-stratigraphic character, age, extent and (where known) thickness of the outcropping and many of the subsurface sedimentary successions of the Arctic Region from 58°-64° to 90° N that may be prospective for hydrocarbon (oil and gas) deposits.  The map also shows structural features that may enhance or detract from the hydrocarbon prospectivity of the mapped sedimentary successions, the distribution of oil and gas fields discovered through 2006, the location of diapirs, impact structures (astroblemes) and kimberlite pipes and the distribution and character of non-prospective terranes.  Synoptic boxes within, or attached to each sedimentary succession shown on the map summarize its first-order stratigraphy and the age and character of basement.  The variable scales, and in many areas the reconnaissance character of the source maps limits the spatial resolution (scale) of the data base to 1:1,000,000 or smaller.

Purpose:

This database and accompanying plot files were prepared as a first step in the Circum-Arctic Resource Appraisal by the World Energy Project of the U.S. Geological Survey (USGS).  The study is part of a Congressionally-mandated evaluation of the World’s hydrocarbon resources by USGS, and was conducted under the leadership of Donald L. Gautier (USGS-Menlo Park, CA). The database depicts the distribution of successions of sedimentary rocks in the Arctic Region that appear to be sufficiently thick to have generated oil and/or natural gas but have not been subjected to degrees of structural deformation or thermal alteration that would have destroyed or dispersed any hydrocarbon deposits that may have formed within them. Most of the near surface and many subsurface sedimentary successions of the Arctic Region are shown although some deeply buried successions were probably not recognized.

Supplemental_Information:

DIGITAL COMPILATION-The map consists of four separate ArcGIS 9.1 geodatabase files, which comprise the Alaska, Barents-Kara, East Siberian, and Greenland quadrants. Map layers are made up of personal geodatabase feature classes and feature datasets. The PDF map layout and map explanation were created in Adobe Illustrator CS3.

BASE MAP- The base map for the digital compilation is The International Bathymetric Chart of the Arctic Ocean (IBCAO), which was downloaded from NOAA web site http://www.ngdc.noaa.gov/mgg/bathymetry/arctic/arctic.html   in ArcView shape file format and converted to ArcInfo coverage for incorporation into the data base.  The bathymetric contours shown by solid lines have contour intervals of 100m from the shoreline to the 200m isobath and of 500 m from the 500m to the 5,500 m isobath.  The 3,800m isobath is shown by dashed lines.  Map projection is North Polar Stereographic with the Standard Parallel at 75°N; Horizontal Datum and Ellipsoid are from the World Geodetic System (WGS 84).  The horizontal position of the isobaths matches that of the IBCAO, which is based on irregularly distributed bathymetric data of regionally variable quality, and therefore provides only a generalized representation of the configuration of the seabed.  Shorelines were downloaded from the World Vector Shoreline (WVS) Coastline Extractor database designed for a scale of 1:5,000,000  http://rimmer.ngdc.noaa.gov. The latitude-longitude grid lines were created in ArcInfo 8.1.

PAMPHLET accompanies map

Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: 11/18/08

Currentness_Reference: publication date

Status:*

Progress: Complete

Maintenance_and_Update_Frequency: As needed

Spatial_Domain:

Bounding_Coordinates:

Alaska quadrant

West_Bounding_Coordinate: -179.99502

East_Bounding_Coordinate: -89.97885

North_Bounding_Coordinate: 89.99453

South_Bounding_Coordinate: 62.40461

Barents-Kara quadrant

West_Bounding_Coordinate: 0.0

East_Bounding_Coordinate: 90.09956

North_Bounding_Coordinate: 89.99453

South_Bounding_Coordinate: 63.16359

East Siberian quadrant

West_Bounding_Coordinate: 89.98856

East_Bounding_Coordinate: 179.99999

North_Bounding_Coordinate: 89.98993

South_Bounding_Coordinate: 62.96711

Greenland quadrant

West_Bounding_Coordinate: -90.00718

East_Bounding_Coordinate: 0.00185

North_Bounding_Coordinate: 89.99839

South_Bounding_Coordinate: 58.02233

Keywords:

Theme:

Theme_Keyword_Thesaurus:  None

Theme_Keyword:  Tectono-stratigraphic

Theme_Keyword:  sedimentary successions

Theme_Keyword:  hydrocarbons

Theme_Keyword:  nonprospective terranes

Theme_Keyword:  Alaska Beaufort Prograded Margin

Theme_Keyword:  Amundsen Basin

Theme_Keyword:  Anadyr Basin

Theme_Keyword:  Anderson Plain Platform

Theme_Keyword:  Arctic Alaska Basin

Theme_Keyword:  Arctic Platform

Theme_Keyword:  Beaufort-Mackenzie Basin

Theme_Keyword:  Canada Basin

Theme_Keyword:  Canadian Arctic Prograded Margin

Theme_Keyword:  Colville Foreland Basin

Theme_Keyword:  Dinkum Graben

Theme_Keyword:  Eagle Foreland Basin

Theme_Keyword:  East Siberian Coastal Plain and Shelf Basin

Theme_Keyword:  Hope Basin

Theme_Keyword:  Kucherov Terrace Margin

Theme_Keyword:  Kugmallit Trough

Theme_Keyword:  Lomonosov Ridge and Marvin Spur

Theme_Keyword:  Long Strait Basin

Theme_Keyword:  Mackenzie Coastal Plain and Shelf Basin

Theme_Keyword:  Makarov Basin

Theme_Keyword:  Marvin Prograded Margin

Theme_Keyword:  Meighan Trough

Theme_Keyword:  North Chukchi Prograded Margin and Ponded Basin

Theme_Keyword:  Northwest Canada Margin Riftogenic Basins

Theme_Keyword:  Northwind Basin

Theme_Keyword:  Northwind Ridge-Detached Fragment of Arctic Platform and Sverdrup Basin

Theme_Keyword:  Norton Basin

Theme_Keyword:  Ogilvie Platform

Theme_Keyword:  Queen Elizabeth Islands Foreland Basin

Theme_Keyword:  Sverdrup Basin

Theme_Keyword:  Wrangel Foreland Basin

Theme_Keyword:  Yukon Flats Basin

Theme_Keyword:  Chaun Basin

Theme_Keyword:  De Long Massif

Theme_Keyword:  East Siberian Sea Basin

Theme_Keyword:  Laptev Shelf

Theme_Keyword:  Lena-Anabar Basin

Theme_Keyword:  Lena Prodelta

Theme_Keyword:  Lena Prograded Margin

Theme_Keyword:  Lomonosov Ridge Basin

Theme_Keyword:  Long Strait Basin

Theme_Keyword:  Lower Kolyma Basin

Theme_Keyword:  Mohn’s Ridge

Theme_Keyword:  Moma Rift System Basins

Theme_Keyword:  Moma-Zyryanka Basin

Theme_Keyword:  Nansen Basin

Theme_Keyword:  North Barents-Kara Prograded Margin

Theme_Keyword:  North Chukchi Prograded Margin

Theme_Keyword:  Olenek-Anabar Anticlise

Theme_Keyword:  Peri-DeLong Basin

Theme_Keyword:  Podvodnikov Basin

Theme_Keyword:  Podvodnikov Prograded Margin

Theme_Keyword:  Priverkhoyansk Basin

Theme_Keyword:  Udzhin-Khastakh-Motorchun Basin

Theme_Keyword:  Vilkitskii Basin

Theme_Keyword:  Vilyui-Kyutyungda Basin

Theme_Keyword:  Western South Taimyr Fold Belt

Theme_Keyword:  Western Zone of Verkhoyansk Fold Belt

Theme_Keyword:  Andree Land Basin

Theme_Keyword:  Barents-Kara Prograded Margin

Theme_Keyword:  Billefjorden Trough

Theme_Keyword:  Bjarmeland Platform

Theme_Keyword:  Bjornoya Basin

Theme_Keyword:  Central Basin

Theme_Keyword:  Finnmark Platform

Theme_Keyword:  Hammerfest Basin

Theme_Keyword:  Harstad Basin

Theme_Keyword:  Helgeland Basin

Theme_Keyword:  Inner Hornsund Trough

Theme_Keyword:  Kureya-Tunguska Basin

Theme_Keyword:  Lofoten Basin

Theme_Keyword:  Mezen' Basin

Theme_Keyword:  More Basin

Theme_Keyword:  More Marginal High

Theme_Keyword:  Nansen Basin

Theme_Keyword:  Nordkapp Basin

Theme_Keyword:  North Barents Basin

Theme_Keyword:  North Barents Platform

Theme_Keyword:  North Kara Basin

Theme_Keyword:  North Kara Platform

Theme_Keyword:  Norweigan Basin

Theme_Keyword:  Polar Urals-Novaya Zemlya Foreland Basin

Theme_Keyword:  Saint Anna Trough

Theme_Keyword:  Schmidt Trough

Theme_Keyword:  Sorvestsnaget Basin

Theme_Keyword:  South Barents Basin

Theme_Keyword:  St. Jonsfjorden Trough

Theme_Keyword:  Taimyr Foreland Basin

Theme_Keyword:  Timan-Pechora Basin

Theme_Keyword:  Tromso Basin

Theme_Keyword:  Trondelag Platforrm

Theme_Keyword:  Utrost Ridge

Theme_Keyword:  Voring Basin

Theme_Keyword:  Voring Marginal High

Theme_Keyword:  West Barents Prograded Margin

Theme_Keyword:  West Siberian-South Kara Basin

Theme_Keyword:  Yenisei-Khatanga Trough

Theme_Keyword:  Yermak Plateau

Theme_Keyword:  Arctic Platform

Theme_Keyword:  Baffin Basin

Theme_Keyword:  Canadian Arctic Prograded Margin

Theme_Keyword:  Danmarkshavn Basin

Theme_Keyword:  East Greenland Margin Riftogenic Basins

Theme_Keyword:  Foxe Basin

Theme_Keyword:  Greenland Basin

Theme_Keyword:  Hudson Strait Basin

Theme_Keyword:  Irminger Basin

Theme_Keyword:  Jameson Land Basin

Theme_Keyword:  Jan Mayen Basin

Theme_Keyword:  Labrador Basin

Theme_Keyword:  Labrador Prograded Margin

Theme_Keyword:  Lincoln Sea Basin

Theme_Keyword:  Lomonosov Ridge

Theme_Keyword:  Morris Jessup Rise

Theme_Keyword:  North Greenland Platform

Theme_Keyword:  North Greenland Prograded Margin

Theme_Keyword:  North Greenland Slope Basin

Theme_Keyword:  Northeast Canada Margin Riftogenic Basin

Theme_Keyword:  Northeast Greenland Prograded Margin

Theme_Keyword:  Queen Elizabeth Islands Foreland basin

Theme_Keyword:  Saglek Riftogenic Basin

Theme_Keyword:  Southeast Greenland Prograded Margin

Theme_Keyword:  Sverdrup Basin

Theme_Keyword:  Thetis Basin

Theme_Keyword:  Wandel Sea Basin

Theme_Keyword:  West Greenland Margin Riftogenic Basin

Theme_Keyword:  West Greenland Prograded Margin

Theme_Keyword:  South Anyui Suture

Theme_Keyword:  Herald Fault Zone

Theme_Keyword:  Kobuk Fault Zone

Theme_Keyword:  Kaltag Fault

Theme_Keyword:  Tintina Fault

Theme_Keyword:  Yukon Fault

Theme_Keyword:  Eskimo Lakes Fault

Theme_Keyword:  Kugmallit Trough

Theme_Keyword:  Amerasia Basin Transform Fault

Theme_Keyword:  Central Taimyr Fold Belt

Theme_Keyword:  South Taimyr Fold Belt

Theme_Keyword:  Kara Massif

Theme_Keyword:  Agapsky Trough

Theme_Keyword:  Bogandsko-Zhdanikhinsky Trough

Theme_Keyword:  Kotuy Aulacogen

Theme_Keyword:  North Siberian Monocline

Theme_Keyword:  Anabar Shield

Theme_Keyword:  Popigai Impact Structure

Theme_Keyword:  Unzhin Aulacogen

Theme_Keyword:  Khastakh Aulacogen

Theme_Keyword:  Kyutyungda Rift

Theme_Keyword:  Sukhan-Motorchun Aulacogen

Theme_Keyword:  Muna Arch

Theme_Keyword:  Olenek High

Theme_Keyword:  Siberian Craton

Theme_Keyword:  Gakkel Ridge

Theme_Keyword:  Marvin Basin

Theme_Keyword:  Anisin-Novosibirsk Basin

Theme_Keyword:  Anisin Rift

Theme_Keyword:  Novosibirsk Rift

Theme_Keyword:  Bel’kov Rift

Theme_Keyword:  East Laptev Basin

Theme_Keyword:  Southwest Laptev Basin

Theme_Keyword:  Ust’ Lena Basin

Theme_Keyword:  South Anyui Deformation

Theme_Keyword:  Chukota Microplate

Theme_Keyword:  Chukchi Fold Belt

Theme_Keyword:  Kolyma-Omolon Superterrane

Theme_Keyword:  Moma Graben

Theme_Keyword:  Upper Selennyakh Graben

Theme_Keyword:  Uyandina Graben

Theme_Keyword:  Chersky Suture

Theme_Keyword:  Tastakh Depression

Theme_Keyword:  Ust’ Yana Graben

Theme_Keyword:  Omoloi Graben

Theme_Keyword:  Vilyui Syneclise

Theme_Keyword:  Verkhoyansk Fold Belt

Theme_Keyword:  Alpha-Mendeleev Large Igneous Province

Theme_Keyword:  West Timan Fault

Theme_Keyword:  Central Timan Fault

Theme_Keyword:  East Timan Fault

Theme_Keyword:  Timanide Orogen

Theme_Keyword:  Shapkin-Yuryakha Fault System

Theme_Keyword:  Kolva Fault System

Theme_Keyword:  Vuktyl Overthrust

Theme_Keyword:  Vuktyl-Polyudov Thrust

Theme_Keyword:  Ilych-Chiksha Fault

Theme_Keyword:  West Uralian Thrust

Theme_Keyword:  Chernyishev Faults

Theme_Keyword:  Varandey Fault System

Theme_Keyword:  Vashutkin-Talotin Thrust

Theme_Keyword:  Central Korotaikha Thrust

Theme_Keyword:  Main West Ural Thrust

Theme_Keyword:  Timanide Deformation

Theme_Keyword:  Trolfjorden-Komagelva Fault Zone

Theme_Keyword:  Iapetus Suture

Theme_Keyword:  Moffen Fault

Theme_Keyword:  Aegir Ridge

Theme_Keyword:  Mohns Ridge

Theme_Keyword:  Iceland-Faroe Large Igneous Province

Theme_Keyword:  Jan Mayen Microcontinent

Theme_Keyword:  Jan Mayen Fracture Zone

Theme_Keyword:  Kolbeinsey Ridge

Theme_Keyword:  Jameson Land Basin

Theme_Keyword:  Liverpool Land Ridge

Theme_Keyword:  Kangerlussaq Basin

Theme_Keyword:  Ammassalik Basin

Theme_Keyword:  Reykjanes Ridge

Theme_Keyword:  Danmarkshavn Ridge

Theme_Keyword:  Caledonian Orogen

Theme_Keyword:  Boreas Basin

Theme_Keyword:  Greenland Ridge

Theme_Keyword:  Greenland Fracture Zone

Theme_Keyword:  Trolle Land Fault

Theme_Keyword:  Lena Trough

Theme_Keyword:  Pearya Exotic Terrane

Theme_Keyword:  Ellesmerian Orogen

Theme_Keyword:  Central Ellesmere Fold Belt

Theme_Keyword:  Ellesmerian Deformation

Theme_Keyword:  Eurekan Deformation

Place:

Place_Keyword: Canada

Place_Keyword: Russia

Place_Keyword: Alaska

Place_Keyword: Arctic Islands

Place_Keyword: Arctic Ocean

Place_Keyword: Chukchi Plateau

Place_Keyword: Mendeleev Ridge

Place_Keyword: North Chukchi Basin

Place_Keyword: Wrangel Island

Place_Keyword: Chukchi Sea

Place_Keyword: Bering Sea

Place_Keyword: Bering Strait

Place_Keyword: Seward Peninsula

Place_Keyword: Axel Heiberg Island

Place_Keyword: Prince Patrick Island

Place_Keyword: Prince of Wales Island

Place_Keyword: King William Island

Place_Keyword: Victoria Island

Place_Keyword: Banks Island

Place_Keyword: Amundsen Gulf

Place_Keyword: Mackenzie River

Place_Keyword: Eastern Siberia

Place_Keyword: East Siberian Sea

Place_Keyword: Servernaya Zemlya

Place_Keyword: Laptev Sea

Place_Keyword: Lena River

Place_Keyword: Lena River Delta

Place_Keyword: Kolyma River

Place_Keyword: New Siberian Islands

Place_Keyword: Western Siberia

Place_Keyword: Eastern Europe

Place_Keyword: Scandinavia

Place_Keyword: Gulf of Bothnia

Place_Keyword: North Atlantic Ocean

Place_Keyword: Barents Sea

Place_Keyword: Kara Sea

Place_Keyword: White Sea

Place_Keyword: Gulf of Ob

Place_Keyword: Svalbard

Place_Keyword: Novya Zemlya

Place_Keyword: Taimyr Peninsula

Place_Keyword: Eurasia Basin

Place_Keyword: Atlantic Ocean

Place_Keyword: Greenland

Place_Keyword: Iceland

Place_Keyword: Greenland Sea

Place_Keyword: Norwegian Sea

Place_Keyword: Labrador

Place_Keyword: Labrador Sea

Place_Keyword: Davis Strait

Place_Keyword: Melville Peninsula

Place_Keyword: Baffin Island

Place_Keyword: Baffin Bay

Place_Keyword: Devon Island

Place_Keyword: Ungava Bay

Place_Keyword: Lincoln Sea

Place_Keyword: Ellesmere Island

Place_Keyword: Alpha Ridge

Place_Keyword: Nares Strait

Place_Keyword: Disko Island

Place_Keyword: Jan Mayen Island

Stratum:

Stratum_Keyword_Thesaurus:  None

Stratum_Keyword: Stable shelf and platform

Stratum_Keyword: Coastal plain and marine shelf

Stratum_Keyword: Basin created by multiple rift and thermo-isostatic (sag) events

Stratum_Keyword: Transtensional rift basin

Stratum_Keyword: Extensional basin on continental crust along the margin of an adjoining ocean basin

Stratum_Keyword: Extensional basin on continental crust along the strike of an adjoining ocean basin

Stratum_Keyword: Extensional basin of undetermined origin

Stratum_Keyword: Foreland basin

Stratum_Keyword: Fore-arc basin

Stratum_Keyword: Progradational sedimentary succession across passive margin of an existing ocean basin

Stratum_Keyword: Progradational sedimentary succession across passive margin of an extinct ocean basin

Stratum_Keyword: Ocean basin

Temporal:

Temporal_Keyword_Thesaurus:  None

Temporal_Keyword: Mesoproterozoic

Temporal_Keyword: Neoproterozoic

Temporal_Keyword: Paleozoic

Temporal_Keyword: Mesozoic

Temporal_Keyword: Cenozoic

Temporal_Keyword: Calymmian

Temporal_Keyword: Ectasian

Temporal_Keyword: Stenian

Temporal_Keyword: Tonian

Temporal_Keyword: Cryogenian

Temporal_Keyword: Ediacaran

Temporal_Keyword:	Cambrian

Temporal_Keyword: Ordovician

Temporal_Keyword: Silurian

Temporal_Keyword: Devonian

Temporal_Keyword: Carboniferous

Temporal_Keyword: Permian

Temporal_Keyword: Triassic

Temporal_Keyword: Jurassic

Temporal_Keyword: Cretaceous

Temporal_Keyword: Paleogene

Temporal_Keyword: Neogene

Temporal_Keyword: Riphean

Temporal_Keyword: Sturtian

Temporal_Keyword: Vendian

Temporal_Keyword: Llandovery

Temporal_Keyword: Wenlock

Temporal_Keyword: Ludlow

Temporal_Keyword: Pridoli

Temporal_Keyword: Mississippian

Temporal_Keyword: Pennsylvanian

Temporal_Keyword: Paleocene

Temporal_Keyword: Eocene

Temporal_Keyword: Oligocene

Temporal_Keyword: Miocene

Temporal_Keyword: Pliocene

Temporal_Keyword: Pleistocene

Temporal_Keyword: Holocene

Access_Constraints: None

Use_Constraints:

Spatial Resolution-Users of this digital map should not violate the spatial resolution of the data because, particularly in offshore areas, where the data are commonly based on small scale or reconnaissance maps or interpreted from potential field data or bathymetry.

Point_of_Contact:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: U.S. Geological Survey World Energy Project

Contact_Person: Arthur Grantz (Consulting Geologist)

Contact_Position: Author

Contact_Address:

Address_Type: mailing and physical address

Address: 930 Van Auken Circle

City: Palo Alto

State_or_Province: CA

Postal_Code: 94303

Country: USA

Data_Set_Credit:

Many geologists and geophysicists who have studied the Arctic Region contributed data and insights  invaluable to the investigation.  We are grateful to John M. Brozena, Vicki Childers and Louis C. Kovacs of the Naval Research Laboratory for access to the laboratory’s aeromagnetic map of the Arctic Ocean, to Seymour W. Laxon (University College, London) and David C. McAdoo NOAA) for early access to their satellite altimetry free-air gravity anomaly map of the Arctic Region to 81° N and to Feliks Persits, (Contractor to USGS) for compiling the bathymetric data base. Data and insights into the regional geology of the Arctic Region were graciously provided by Hugh Balkwill (PetroCanada), Steven Bergman (Shell Oil Company), K. J. Bird (U.S. Geological Survey), Grigory Bondarenko (Geologic Institute, Russian Academy of Sciences), Harald Brekke (Norwegian Petroleum Directorate), Jonathan Bujak (Bujak Research International), J. A. Chalmers and Finn Dalhoff (Geological Survey of Denmark and Greenland), Ashton E. Embry III (Geological Survey of Canada), Dieter Franke (Federal Institute for Geosciences and Natural Resources (BGR) Germany), Lisa M. Gahagen (University of Texas), Donald L. Gautier (U.S. Geological Survey), David G. Gee (Uppsala University), Christopher Harrison (Geological Survey of Canada), David Houseknecht (U.S. Geological Survey), Nina Lebedeva-Ivanova (Uppsala University), H. Ruth Jackson (Geological Survey of Canada, Martin Jakobsson (Stockholm University) Wilfried Jokat (Alfred Wegener Institute), , Viktor Khain (Institute for the Lithosphere, Russian Academy of Sciences), Timothy R. Klett (USGS), Yngve Kristoffersen (University of Bergen), Lawrence A. Lawver (University of Texas), Peter J. McCabe (U.S. Geological Survey), Michael B. Mickey (Micropaleo Consultants, Inc.), Elizabeth L. Miller (Stanford University),  Anatoly Nikishin (Moscow State University), Kirk Osadetz (Geological Survey of Canada), Victoria L. Pease (Stockholm University), Christopher J. Potter (U.S. Geological Survey), Andei V.Prokopiev (Diamons and Precious Metal Gedology Institute, Yakutsk, R.F.),  Kai Srenson and Lars Stemmerik (GUES), David B. Stone (University of Alaska) and Gregory Ulmishek (U.S. Geological Survey).

Funding for this report was funded by the World Energy Project of the U.S. Geological Survey.

The map and Explanatory Pamphlet were reviewed by Arthur R. Green (ExxonMobil, retired) and David W. Scholl (U.S. Geological Survey, Scientist Emeritus) and Albert W. Bally(Rice University) for the U.S. Geological Survey and by B. J. Fossum (ConocoPhillips) and four anonymous reviewers for the American Association of Petroleum Geologists.  We gratefully acknowledge the thoughtful comments and suggestions provided by the reviewers.

Security_Information:

Security_Classification_System: None

Security_Classification:  Unclassified

Security_Handling_Description: None

Native_Data_Set_Environment:

Microsoft Windows XP Version 5.1 (Build 2600) Service Pack 2; ESRI ArcCatalog 9.1.0.722

Data_Quality_Information:

Lineage:

Process_Step:

Process_Description:

Source_Used_Citation_Abbreviation:

Spatial_Data_Organization_Information:

Direct_Spatial_Reference_Method: Vector

Point_and_Vector_Object_Information:

SDTS_Terms_Description:

SDTS_Point_and_Vector_Object_Type:  Point

SDTS_Point_and_Vector_Object_Type:  String

SDTS_Point_and_Vector_Object_Type:  GT-polygon composed of chains

Spatial_Reference_Information:

Coordinate_System_Definition:

Clarke 1866 Stereographic North Pole

Projection: Stereographic North Pole

False Easting: 0.000000

False Northing: 0.000000

Central Meridian: 180.00000

Standard Parallel 1: 75.00000

GCS Clarke 1866

Planar Distance Units: meters

ALASKA QUADRANT FEATURE CLASSES:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: AK_surface_sedimentary_successions

Entity_Type_Definition: Geologic units.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: sedimentary_succession

Attribute_Definition: Geologic unit description.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Basin created by multiple rift and thermo-isostatic (sag) events

Enumerated_Domain_Value_Definition: The extension that created the basin thinned, but did not sever, the underlying continental crust. Typically consist of both siliciclastic and carbonate deposits and in places evaporites

Enumerated_Domain:

Enumerated_Domain_Value: Coastal plain and marine shelf

Enumerated_Domain_Value_Definition: Epicontinental nonmarine and marine shelf deposits of Mesozoic and (or) Cenozoic age, typically no more than 1 or 2 km thick. Overlain by eupelagic deposits on Lomonosov Ridge, Marvin Spur and probably Yermak Plateau

Enumerated_Domain:

Enumerated_Domain_Value: Extensional basin on continental crust along the margin of an adjoining ocean basin. The stretching that created the extensional basin was a precursor to seafloor spreading in the ocean basin

Enumerated_Domain_Value_Definition: Contains mainly siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Fore-arc basin

Enumerated_Domain_Value_Definition: Basin formed between an accretionary wedge adjacent to a subduction zone and its associated island arc.  Contains siliciclastic and volcaniclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Foreland basin

Enumerated_Domain_Value_Definition: Basin created by flexural, sedimentary and thrust sheet loading of continental crust in front of a convergence zone. Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Ocean basin

Enumerated_Domain_Value_Definition: Eupelagic, hemipelagic or turbidite deposits overlying mid-ocean ridge basalt (MORB) or ocean-continent transitional (extended continental) crust

Enumerated_Domain:

Enumerated_Domain_Value: Progradational sedimentary succession across predominantly rifted passive margin of an existing ocean basin

Enumerated_Domain_Value_Definition: Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Progradational sedimentary succession across predominantly strike-slip margin of an existing ocean basin

Enumerated_Domain_Value_Definition: Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Stable shelf and platform

Enumerated_Domain_Value_Definition: Commonly contain carbonate as well as clastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Transtensional rift basin

Enumerated_Domain_Value_Definition: Basin formed above a broad transcurrent fault system with a large component of extension.  Contains siliciclastic deposits

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: AK_subsurface_sedimentary_successions

Entity_Type_Definition: Geologic units.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain:

Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: sedimentary_succession

Attribute_Definition: Geologic unit description.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

(Numeric values in parentheses contained in the geodatabase feature class, which refer to the angle of the stripes in the fill pattern, have been omitted from the following domain value descriptions.)

Enumerated_Domain:

Enumerated_Domain_Value: Basin created by multiple rift and thermo-isostatic (sag) events, underlying

Enumerated_Domain_Value_Definition: The extension that created the basin thinned, but did not sever, the underlying continental crust. Typically consist of both siliciclastic and carbonate deposits and in places evaporites

Enumerated_Domain:

Enumerated_Domain_Value: Extensional basin on continental crust along the margin of an adjoining ocean basin. The stretching that created the extensional basin was a precursor to seafloor spreading in the ocean basin, underlying

Enumerated_Domain_Value_Definition: Contains mainly siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Foreland basin, underlying

Enumerated_Domain_Value_Definition: Basin created by flexural, sedimentary and thrust sheet loading of continental crust in front of a convergence zone. Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Ocean basin, underlying

Enumerated_Domain_Value_Definition: Eupelagic, hemipelagic or turbidite deposits overlying mid-ocean ridge basalt (MORB) or ocean-continent transitional (extended continental) crust

Enumerated_Domain:

Enumerated_Domain_Value: Stable shelf and platform, underlying

Enumerated_Domain_Value_Definition: Commonly contain carbonate as well as clastic deposits

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: AK_boundaries_and_structural_axes

Entity_Type_Definition: Geologic line types.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: line_feature

Attribute_Definition: Geologic line feature.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Active sea-floor spreading axis, inferred

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Axis of structural arch expressed in configuration of basement, concealed

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Axis of structural arch expressed in configuration of basement, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Axis of structural trough expressed in configuration of basement, concealed

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Axis of structural trough expressed in configuration of basement, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Axis of structural trough expressed in configuration of basement, inferred

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary between geographic subdivisions of a sedimentary succession

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary between mid-ocean-ridge basalt (MORB) and ocean-continent transitional crust (OCT)

Enumerated_Domain_Value_Definition: Approximated from aeromagnetic or seismic data, shown locally

Enumerated_Domain:

Enumerated_Domain_Value: Boundary between subdivisions of a sedimentary succession

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Continent-ocean boundary

Enumerated_Domain_Value_Definition: Approximated from aeromagnetic or seismic data, shown locally

Enumerated_Domain:

Enumerated_Domain_Value: Depositional boundary of a sedimentary succession, concealed

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Depositional boundary of a sedimentary succession, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Depositional boundary of a sedimentary succession, inferred

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Extinct sea-floor spreading axis, inferred

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Passive continental margin hinge line, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Passive continental margin hinge line, inferred

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Structural arch not involving basement, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Structural arch not involving basement, inferred

Enumerated_Domain_Value_Definition:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: AK_faulted_boundary_of_a_sedimentary_succession

Entity_Type_Definition: Faulted boundary of a sedimentary succession or its subdivisions.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: line_feature

Attribute_Definition: Geologic line feature.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Faulted boundary of a sedimentary succession or its subdivisions                            							Enumerated_Domain_Value_Definition: Ornaments as for Faults and Structural Fronts

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: AK_faults_and_structural_fronts

Entity_Type_Definition: Fault line types.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: name

Attribute_Definition: Name of fault or structural front.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Attribute:

Attribute_Label: fault_type

Attribute_Definition: Fault line and structural front types.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Back thrust, generalized

Enumerated_Domain_Value_Definition: Saw teeth face upper plate

Enumerated_Domain:

Enumerated_Domain_Value: Deformation front, concealed

Enumerated_Domain_Value_Definition: Separates folded from undeformed strata. Ornament faces folded strata

Enumerated_Domain:

Enumerated_Domain_Value: Deformation front, generalized

Enumerated_Domain_Value_Definition: Separates folded from undeformed strata. Ornament faces folded strata

Enumerated_Domain:

Enumerated_Domain_Value: Deformation front, inferred

Enumerated_Domain_Value_Definition: Separates folded from undeformed strata. Ornament faces folded strata

Enumerated_Domain:

Enumerated_Domain_Value: Deformational boundary, concealed

Enumerated_Domain_Value_Definition: Separates strata with numerous thrust faults and associated folds from folded strata with relatively few faults.

Ornament faces the more intensely deformed strata

Enumerated_Domain:

Enumerated_Domain_Value: Deformational boundary, generalized

Enumerated_Domain_Value_Definition: Separates strata with numerous thrust faults and associated folds from folded strata with relatively few faults. Ornament faces the more intensely deformed strata

Enumerated_Domain:

Enumerated_Domain_Value: Deformational boundary, inferred

Enumerated_Domain_Value_Definition: Separates strata with numerous thrust faults and associated folds from folded strata with relatively few faults. Ornament faces the more intensely deformed strata

Enumerated_Domain:

Enumerated_Domain_Value: Detachment fault, concealed

Enumerated_Domain_Value_Definition: Separates strata with numerous thrust faults and associated folds from folded strata with relatively few faults. Ornament faces the more intensely deformed strata

Enumerated_Domain:

Enumerated_Domain_Value: Fault, concealed

Enumerated_Domain_Value_Definition: Character not known

Enumerated_Domain:

Enumerated_Domain_Value: Fault, generalized

Enumerated_Domain_Value_Definition: Character not known

Enumerated_Domain:

Enumerated_Domain_Value: Fault, inferred

Enumerated_Domain_Value_Definition: Character not known

Enumerated_Domain:

Enumerated_Domain_Value: Normal fault, concealed

Enumerated_Domain_Value_Definition: Hachures face hanging wall

Enumerated_Domain:

Enumerated_Domain_Value: Normal fault, generalized

Enumerated_Domain_Value_Definition: Hachures face hanging wall

Enumerated_Domain:

Enumerated_Domain_Value: Normal fault, inferred

Enumerated_Domain_Value_Definition: Hachures face hanging wall

Enumerated_Domain:

Enumerated_Domain_Value: Strike-slip fault, concealed

Enumerated_Domain_Value_Definition: Arrows show relative displacement

Enumerated_Domain:

Enumerated_Domain_Value: Strike-slip fault, generalized

Enumerated_Domain_Value_Definition: Arrows show relative displacement

Enumerated_Domain:

Enumerated_Domain_Value: Strike-slip fault, inferred

Enumerated_Domain_Value_Definition: Arrows show relative displacement

Enumerated_Domain:

Enumerated_Domain_Value: Structural front, concealed

Enumerated_Domain_Value_Definition: Separates intensely folded, faulted or metamorphosed strata from strata containing only open folds.  Ornament faces the more intensely deformed terrane, which is considered to lack significant hydrocarbon potential

Enumerated_Domain:

Enumerated_Domain_Value: Structural front, generalized

Enumerated_Domain_Value_Definition: Separates intensely folded, faulted or metamorphosed strata from strata containing only open folds.  Ornament faces the more intensely deformed terrane, which is considered to lack significant hydrocarbon potential

Enumerated_Domain:

Enumerated_Domain_Value: Structural front, inferred

Enumerated_Domain_Value_Definition: Separates intensely folded, faulted or metamorphosed strata from strata containing only open folds.  Ornament faces the more intensely deformed terrane, which is considered to lack significant hydrocarbon potential

Enumerated_Domain:

Enumerated_Domain_Value: Suture, concealed

Enumerated_Domain_Value_Definition: Boundary between paleo-continents formerly separated by an ocean basin. “O” indicates presence of ophiolite.  Saw teeth indicate suture is a thrust fault

Enumerated_Domain:

Enumerated_Domain_Value: Thrust or reverse fault, generalized

Enumerated_Domain_Value_Definition: Saw teeth face upper plate

Enumerated_Domain:

Enumerated_Domain_Value: Thrust or reverse fault, inferred

Enumerated_Domain_Value_Definition: Saw teeth face upper plate

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: AK_nonprospective_terranes

Entity_Type_Definition: Nonprospective terrane types.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: line_feature

Attribute_Definition: Nonprospective terrane line feature.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Boundaries of Alpha-Mendeleev and Iceland-Faroe Large Igneous Provinces (LIP), based on aeromagnetic data, concealed

Enumerated_Domain_Value_Definition: The Alpha-Mendeleev LIP is overlain by 0 to >1.5 km of eupelagic or hemipelagic sediment and bedrock samples at one locality consist of alkali basalt 120 to 90 Ma

Enumerated_Domain:

Enumerated_Domain_Value: Boundaries of Alpha-Mendeleev and Iceland-Faroe Large Igneous Provinces (LIP), based on aeromagnetic data, generalized

Enumerated_Domain_Value_Definition: The Alpha-Mendeleev LIP is overlain by 0 to >1.5 km of eupelagic or hemipelagic sediment and bedrock samples at one locality consist of alkali basalt 120 to 90 Ma

Enumerated_Domain:

Enumerated_Domain_Value: Boundaries of Alpha-Mendeleev and Iceland-Faroe Large Igneous Provinces (LIP), based on aeromagnetic data, inferred

Enumerated_Domain_Value_Definition: The Alpha-Mendeleev LIP is overlain by 0 to >1.5 km of eupelagic or hemipelagic sediment and bedrock samples at one locality consist of alkali basalt 120 to 90 Ma

Enumerated_Domain:

Enumerated_Domain_Value: Boundaries of Alpha-Mendeleev and Iceland-Faroe Large Igneous Provinces (LIP), based on aeromagnetic data, no line

Enumerated_Domain_Value_Definition: The Alpha-Mendeleev LIP is overlain by 0 to >1.5 km of eupelagic or hemipelagic sediment and bedrock samples at one locality consist of alkali basalt 120 to 90 Ma

Enumerated_Domain:

Enumerated_Domain_Value: Boundary of ridge of continental crust in an ocean basin, concealed

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary of ridge of continental crust in an ocean basin, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary of ridge of continental crust in an ocean basin, no line

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Faulted boundary of ridge of continental crust in an ocean basin, concealed

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Faulted boundary of ridge of continental crust in an ocean basin, strike-slip concealed

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Faulted boundary of ridge of continental crust in an ocean basin, thrust concealed

Enumerated_Domain_Value_Definition:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: AK_oil_and_gas_fields

Entity_Type_Definition: Oil and gas field locations.

Entity_Type_Definition_Source: IHS Energy

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: FIELD_NAME

Attribute_Definition: Names of oil and gas fields.

Attribute_Definition_Source: IHS Energy

Attribute_Domain_Values:

Attribute:

Attribute_Label: HC_TYPE

Attribute_Definition: Hydrocarbon types of field locations.

Attribute_Definition_Source: IHS Energy

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Gas

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Gas and condensate

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Gas and oil

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Gas, condensate and oil

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Oil

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Oil and gas

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Oil, gas and condensate

Enumerated_Domain_Value_Definition:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: AK_isopachs

Entity_Type_Definition: Lines indicating thickness of stratigraphic 	interval, in km.

Entity_Type_Definition_Source:

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Attribute:

Attribute_Label: thickness

Attribute_Definition: Thickness of stratigraphic interval, in km.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Attribute:

Attribute_Label: line_type

Attribute_Definition: Isopach line symbol type

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: AK_geophysical_profiles

Entity_Type_Definition: Profiles showing estimated thickness of 	sedimentary strata, in km.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: AK_diapirs

Entity_Type_Definition: Diapir locations.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: type

Attribute_Definition: Sample type.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Diapir

Enumerated_Domain_Value_Definition: Evaporite, locally shale, undifferentiated. Triangles show locality only

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: AK_impact_structure

Entity_Type_Definition: Impact structure location.

Entity_Type_Definition_Source:

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: type

Attribute_Definition: Impact structure.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Impact structure

Enumerated_Domain_Value_Definition: astrobleme

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: AK_LIP_interior

Entity_Type_Definition: Areas within Large Igneous Province (LIP) boundaries

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

BARENTS-KARA QUADRANT FEATURE CLASSES:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: BK_surface_sedimentary_successions

Entity_Type_Definition: Geologic units.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: sedimentary_succession

Attribute_Definition: Geologic unit description.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Basin created by multiple rift and thermo-isostatic (sag) events

Enumerated_Domain_Value_Definition: The extension that created the basin thinned, but did not sever, the underlying continental crust. Typically consist of both siliciclastic and carbonate deposits and in places evaporates

Enumerated_Domain:

Enumerated_Domain_Value: Coastal plain and marine shelf

Enumerated_Domain_Value_Definition: Epicontinental nonmarine and marine shelf deposits of Mesozoic and (or) Cenozoic age, typically no more than 1 or 2 km thick. Overlain by eupelagic deposits on Lomonosov Ridge, Marvin Spur and probably Yermak Plateau

Enumerated_Domain:

Enumerated_Domain_Value: Extensional basin on continental crust along the margin of an adjoining ocean basin. The stretching that created the extensional basin was a precursor to seafloor spreading in the ocean basin

Enumerated_Domain_Value_Definition: Contains mainly siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Foreland basin

Enumerated_Domain_Value_Definition: Basin created by flexural, sedimentary and thrust sheet loading of continental crust in front of a convergence zone. Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Ocean basin

Enumerated_Domain_Value_Definition: Eupelagic, hemipelagic or turbidite deposits overlying mid-ocean ridge basalt (MORB) or ocean-continent transitional (extended continental) crust

Enumerated_Domain:

Enumerated_Domain_Value: Progradational sedimentary succession across predominantly rifted passive margin of an existing ocean basin

Enumerated_Domain_Value_Definition: Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Stable shelf and platform

Enumerated_Domain_Value_Definition: Commonly contain carbonate as well as clastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Transtensional rift basin

Enumerated_Domain_Value_Definition: Basin formed above a broad transcurrent fault system with a large component of extension.  Contains siliciclastic deposits

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: BK_subsurface_sedimentary_successions

Entity_Type_Definition: Geologic units.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain:

Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: sedimentary_succession

Attribute_Definition: Geologic unit description.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

(Numeric values in parentheses contained in the geodatabase feature class, which refer to the angle of the stripes in the fill pattern, have been omitted from the following domain value descriptions.)

Enumerated_Domain:

Enumerated_Domain_Value: Basin created by multiple rift and thermo-isostatic (sag) events, underlying

Enumerated_Domain_Value_Definition: The extension that created the basin thinned, but did not sever, the underlying continental crust. Typically consist of both siliciclastic and carbonate deposits and in places evaporates

Enumerated_Domain:

Enumerated_Domain_Value: Extensional basin on continental crust along the margin of an adjoining ocean basin. The stretching that created the extensional basin was a precursor to seafloor spreading in the ocean basin, underlying

Enumerated_Domain_Value_Definition: Contains mainly siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Foreland basin, underlying

Enumerated_Domain_Value_Definition: Basin created by flexural, sedimentary and thrust sheet loading of continental crust in front of a convergence zone. Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Ocean basin, underlying

Enumerated_Domain_Value_Definition: Eupelagic, hemipelagic or turbidite deposits overlying mid-ocean ridge basalt (MORB) or ocean-continent transitional (extended continental) crust

Enumerated_Domain:

Enumerated_Domain_Value: Stable shelf and platform, underlying

Enumerated_Domain_Value_Definition: Commonly contain carbonate as well as clastic deposits

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: BK_boundaries_and_structural_axes

Entity_Type_Definition: Geologic line types.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: line_feature

Attribute_Definition: Geologic line feature.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Active sea-floor spreading axis, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Axis of structural arch expressed in configuration of basement, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Axis of structural trough expressed in configuration of basement, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Basin boundary

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Basin boundary, inferred

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary between geographic subdivisions of a sedimentary succession

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary between subdivisions of a sedimentary succession

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Continent-ocean boundary

Enumerated_Domain_Value_Definition: Approximated from aeromagnetic or seismic data, shown locally

Enumerated_Domain:

Enumerated_Domain_Value: Depositional boundary of a sedimentary succession, concealed

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Depositional boundary of a sedimentary succession, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Depositional boundary of a sedimentary succession, inferred

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Structural arch not involving basement, generalized

Enumerated_Domain_Value_Definition:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: BK_faulted_boundary_of_a_sedimentary_succession

Entity_Type_Definition: Faulted boundary of a sedimentary succession or its subdivisions.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: line_feature

Attribute_Definition: Geologic line feature.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Faulted boundary of a sedimentary succession or its subdivisions                            							Enumerated_Domain_Value_Definition: Ornaments as for Faults and Structural Fronts

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: BK_faults_and_structural_fronts

Entity_Type_Definition: Fault line types.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: name

Attribute_Definition: Name of fault or structural front.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Attribute:

Attribute_Label: fault_type

Attribute_Definition: Fault line and structural front types.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Deformation front, generalized

Enumerated_Domain_Value_Definition: Separates folded from undeformed strata. Ornament faces folded strata

Enumerated_Domain:

Enumerated_Domain_Value: Fault, concealed

Enumerated_Domain_Value_Definition: Character not known

Enumerated_Domain:

Enumerated_Domain_Value: Fault, generalized

Enumerated_Domain_Value_Definition: Character not known

Enumerated_Domain:

Enumerated_Domain_Value: Fault, inferred

Enumerated_Domain_Value_Definition: Character not known

Enumerated_Domain:

Enumerated_Domain_Value: Normal fault, concealed

Enumerated_Domain_Value_Definition: Hachures face hanging wall

Enumerated_Domain:

Enumerated_Domain_Value: Normal fault, generalized

Enumerated_Domain_Value_Definition: Hachures face hanging wall

Enumerated_Domain:

Enumerated_Domain_Value: Normal fault, inferred

Enumerated_Domain_Value_Definition: Hachures face hanging wall

Enumerated_Domain:

Enumerated_Domain_Value: Strike-slip fault, generalized

Enumerated_Domain_Value_Definition: Arrows show relative displacement

Enumerated_Domain:

Enumerated_Domain_Value: Structural front, concealed

Enumerated_Domain_Value_Definition: Separates intensely folded, faulted or metamorphosed strata from strata containing only open folds.  Ornament faces the more intensely deformed terrane, which is considered to lack significant hydrocarbon potential

Enumerated_Domain:

Enumerated_Domain_Value: Structural front, generalized

Enumerated_Domain_Value_Definition: Separates intensely folded, faulted or metamorphosed strata from strata containing only open folds.  Ornament faces the more intensely deformed terrane, which is considered to lack significant hydrocarbon potential

Enumerated_Domain:

Enumerated_Domain_Value: Structural front, inferred

Enumerated_Domain_Value_Definition: Separates intensely folded, faulted or metamorphosed strata from strata containing only open folds.  Ornament faces the more intensely deformed terrane, which is considered to lack significant hydrocarbon potential

Enumerated_Domain:

Enumerated_Domain_Value: Suture thrust fault, generalized

Enumerated_Domain_Value_Definition: Boundary between paleo-continents formerly separated by an ocean basin. “O” indicates presence of ophiolite.  Saw teeth indicate suture is a thrust fault

Enumerated_Domain:

Enumerated_Domain_Value: Suture thrust fault, concealed

Enumerated_Domain_Value_Definition: Boundary between paleo-continents formerly separated by an ocean basin. “O” indicates presence of ophiolite.  Saw teeth indicate suture is a thrust fault

Enumerated_Domain:

Enumerated_Domain_Value: Thrust or reverse fault, concealed

Enumerated_Domain_Value_Definition: Saw teeth face upper plate

Enumerated_Domain:

Enumerated_Domain_Value: Thrust or reverse fault, generalized

Enumerated_Domain_Value_Definition: Saw teeth face upper plate

Enumerated_Domain:

Enumerated_Domain_Value: Thrust or reverse fault, inferred

Enumerated_Domain_Value_Definition: Saw teeth face upper plate

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: BK_nonprospective_terranes

Entity_Type_Definition: Nonprospective terrane types.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: line_feature

Attribute_Definition: Nonprospective terrane line feature.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary of mid-ocean ridge basalts (MORB) at the seabed, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary of ridge of continental crust in an ocean basin, no line

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Faulted boundary of ridge of continental crust in an ocean basin, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Faulted boundary of ridge of continental crust in an ocean basin, inferred

Enumerated_Domain_Value_Definition:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: BK_oil_and_gas_fields

Entity_Type_Definition: Oil and gas field locations.

Entity_Type_Definition_Source: IHS Energy

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: FIELD_NAME

Attribute_Definition: Names of oil and gas fields.

Attribute_Definition_Source: IHS Energy

Attribute_Domain_Values:

Attribute:

Attribute_Label: HC_TYPE

Attribute_Definition: Hydrocarbon types of field locations.

Attribute_Definition_Source: IHS Energy

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Gas

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Gas and condensate

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Gas and oil

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Gas, condensate and oil

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Oil

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Oil and gas

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Oil, gas and condensate

Enumerated_Domain_Value_Definition:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: BK_isopachs

Entity_Type_Definition: Lines indicating thickness of stratigraphic 	interval, in km.

Entity_Type_Definition_Source:

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Attribute:

Attribute_Label: thickness

Attribute_Definition: Thickness of stratigraphic interval, in km.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Attribute:

Attribute_Label: line_type

Attribute_Definition: Isopach line symbol type

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: BK_misc_area_features

Entity_Type_Definition: Diapir and impact structure areas.

Entity_Type_Definition_Source:

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: type

Attribute_Definition: Area type.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Diapir

Enumerated_Domain_Value_Definition: Evaporite, locally shale, undifferentiated

Enumerated_Domain:

Enumerated_Domain_Value: Impact structure

Enumerated_Domain_Value_Definition: astrobleme

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: BK_igneous_rocks

Entity_Type_Definition: Igneous rock areas.

Entity_Type_Definition_Source:

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: type

Attribute_Definition: Igneous rock type.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Igneous intrusions

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Intercalated volcanic rocks

Enumerated_Domain_Value_Definition:

EAST SIBERIAN QUADRANT FEATURE CLASSES:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: ES_surface_sedimentary_successions

Entity_Type_Definition: Geologic units.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: sedimentary_succession

Attribute_Definition: Geologic unit description.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Basin created by multiple rift and thermo-isostatic (sag) events

Enumerated_Domain_Value_Definition: The extension that created the basin thinned, but did not sever, the underlying continental crust. Typically consist of both siliciclastic and carbonate deposits and in places evaporates

Enumerated_Domain:

Enumerated_Domain_Value: Coastal plain and marine shelf

Enumerated_Domain_Value_Definition: Epicontinental nonmarine and marine shelf deposits of Mesozoic and (or) Cenozoic age, typically no more than 1 or 2 km thick. Overlain by eupelagic deposits on Lomonosov Ridge, Marvin Spur and probably Yermak Plateau

Enumerated_Domain:

Enumerated_Domain_Value: Extensional basin of undetermined origin

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Extensional basin on continental crust along the strike of an adjoining ocean basin. The stretching that created the extensional basin predated, and persisted throughout, seafloor spreading in the ocean basin

Enumerated_Domain_Value_Definition: Contains mainly siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Fore-arc basin

Enumerated_Domain_Value_Definition: Basin formed between an accretionary wedge adjacent to a subduction zone and its associated island arc.  Contains siliciclastic and volcaniclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Foreland basin

Enumerated_Domain_Value_Definition: Basin created by flexural, sedimentary and thrust sheet loading of continental crust in front of a convergence zone. Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Ocean basin

Enumerated_Domain_Value_Definition: Eupelagic, hemipelagic or turbidite deposits overlying mid-ocean ridge basalt (MORB) or ocean-continent transitional (extended continental) crust

Enumerated_Domain:

Enumerated_Domain_Value: Progradational sedimentary succession across predominantly rifted passive margin of an existing ocean basin

Enumerated_Domain_Value_Definition: Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Progradational sedimentary succession across predominantly strike-slip margin of an existing ocean basin

Enumerated_Domain_Value_Definition: Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Progradational sedimentary succession across passive margin of an extinct ocean basin

Enumerated_Domain_Value_Definition: Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Stable shelf and platform

Enumerated_Domain_Value_Definition: Commonly contain carbonate as well as clastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Transtensional rift basin

Enumerated_Domain_Value_Definition: Basin formed above a broad transcurrent fault system with a large component of extension.  Contains siliciclastic deposits

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: ES_subsurface_sedimentary_successions

Entity_Type_Definition: Geologic units.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain:

Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: sedimentary_succession

Attribute_Definition: Geologic unit description.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

(Numeric values in parentheses contained in the geodatabase feature class, which refer to the angle of the stripes in the fill pattern, have been omitted from the following domain value descriptions.)

Enumerated_Domain:

Enumerated_Domain_Value: Basin created by multiple rift and thermo-isostatic (sag) events, underlying

Enumerated_Domain_Value_Definition: The extension that created the basin thinned, but did not sever, the underlying continental crust. Typically consist of both siliciclastic and carbonate deposits and in places evaporates

Enumerated_Domain:

Enumerated_Domain_Value: Extensional basin on continental crust along the strike of an adjoining ocean basin. The stretching that created the extensional basin predated, and persisted throughout, seafloor spreading in the ocean basin, underlying

Enumerated_Domain_Value_Definition: Contains mainly siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Foreland basin, underlying

Enumerated_Domain_Value_Definition: Basin created by flexural, sedimentary and thrust sheet loading of continental crust in front of a convergence zone. Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Ocean basin, underlying

Enumerated_Domain_Value_Definition: Eupelagic, hemipelagic or turbidite deposits overlying mid-ocean ridge basalt (MORB) or ocean-continent transitional (extended continental) crust

Enumerated_Domain:

Enumerated_Domain_Value: Progradational sedimentary succession across passive margin of an extinct ocean basin, underlying

Enumerated_Domain_Value_Definition: Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Stable shelf and platform, underlying

Enumerated_Domain_Value_Definition: Commonly contain carbonate as well as clastic deposits

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: ES_boundaries_and_structural_axes

Entity_Type_Definition: Geologic line types.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: line_feature

Attribute_Definition: Geologic line feature.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Active sea-floor spreading axis, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Axis of structural arch expressed in configuration of basement, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary between geographic subdivisions of a sedimentary succession

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary between mid-ocean-ridge basalt (MORB) and ocean-continent transitional crust (OCT)

Enumerated_Domain_Value_Definition: Approximated from aeromagnetic or seismic data, shown locally

Enumerated_Domain:

Enumerated_Domain_Value: Boundary between subdivisions of a sedimentary succession

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Continent-ocean boundary

Enumerated_Domain_Value_Definition: Approximated from aeromagnetic or seismic data, shown locally

Enumerated_Domain:

Enumerated_Domain_Value: Depositional boundary of a sedimentary succession, concealed

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Depositional boundary of a sedimentary succession, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Depositional boundary of a sedimentary succession, inferred

Enumerated_Domain_Value_Definition:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: ES_faulted_boundary_of_a_sedimentary_succession

Entity_Type_Definition: Faulted boundary of a sedimentary succession or its subdivisions.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: line_feature

Attribute_Definition: Geologic line feature.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Faulted boundary of a sedimentary succession or its subdivisions                            							Enumerated_Domain_Value_Definition: Ornaments as for Faults and Structural Fronts

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: ES_faults_and_structural_fronts

Entity_Type_Definition: Fault line types.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: name

Attribute_Definition: Name of fault or structural front.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Attribute:

Attribute_Label: fault_type

Attribute_Definition: Fault line and structural front types.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Deformation front, concealed

Enumerated_Domain_Value_Definition: Separates folded from undeformed strata. Ornament faces folded strata

Enumerated_Domain:

Enumerated_Domain_Value: Deformation front, generalized

Enumerated_Domain_Value_Definition: Separates folded from undeformed strata. Ornament faces folded strata

Enumerated_Domain:

Enumerated_Domain_Value: Deformation front, inferred

Enumerated_Domain_Value_Definition: Separates folded from undeformed strata. Ornament faces folded strata

Enumerated_Domain:

Enumerated_Domain_Value: Deformational boundary, generalized

Enumerated_Domain_Value_Definition: Separates strata with numerous thrust faults and associated folds from folded strata with relatively few faults.

Ornament faces the more intensely deformed strata

Enumerated_Domain:

Enumerated_Domain_Value: Detachment fault, concealed

Enumerated_Domain_Value_Definition: Separates strata with numerous thrust faults and associated folds from folded strata with relatively few faults.

Ornament faces the more intensely deformed strata

Enumerated_Domain:

Enumerated_Domain_Value: Detachment fault, generalized

Enumerated_Domain_Value_Definition: Separates strata with numerous thrust faults and associated folds from folded strata with relatively few faults.

Ornament faces the more intensely deformed strata

Enumerated_Domain:

Enumerated_Domain_Value: Detachment fault, inferred

Enumerated_Domain_Value_Definition: Separates strata with numerous thrust faults and associated folds from folded strata with relatively few faults.

Ornament faces the more intensely deformed strata

Enumerated_Domain:

Enumerated_Domain_Value: Fault, concealed

Enumerated_Domain_Value_Definition: Character not known

Enumerated_Domain:

Enumerated_Domain_Value: Fault, generalized

Enumerated_Domain_Value_Definition: Character not known

Enumerated_Domain:

Enumerated_Domain_Value: Fault, inferred

Enumerated_Domain_Value_Definition: Character not known

Enumerated_Domain:

Enumerated_Domain_Value: Normal fault, concealed

Enumerated_Domain_Value_Definition: Hachures face hanging wall

Enumerated_Domain:

Enumerated_Domain_Value: Normal fault, generalized

Enumerated_Domain_Value_Definition: Hachures face hanging wall

Enumerated_Domain:

Enumerated_Domain_Value: Normal fault, inferred

Enumerated_Domain_Value_Definition: Hachures face hanging wall

Enumerated_Domain:

Enumerated_Domain_Value: Strike-slip fault, concealed

Enumerated_Domain_Value_Definition: Arrows show relative displacement

Enumerated_Domain:

Enumerated_Domain_Value: Strike-slip fault, inferred

Enumerated_Domain_Value_Definition: Arrows show relative displacement

Enumerated_Domain:

Enumerated_Domain_Value: Structural front, generalized

Enumerated_Domain_Value_Definition: Separates intensely folded, faulted or metamorphosed strata from strata containing only open folds.  Ornament faces the more intensely deformed terrane, which is considered to lack significant hydrocarbon potential

Enumerated_Domain:

Enumerated_Domain_Value: Structural front, inferred

Enumerated_Domain_Value_Definition: Separates intensely folded, faulted or metamorphosed strata from strata containing only open folds.  Ornament faces the more intensely deformed terrane, which is considered to lack significant hydrocarbon potential

Enumerated_Domain:

Enumerated_Domain_Value: Suture thrust fault, generalized

Enumerated_Domain_Value_Definition: Boundary between paleo-continents formerly separated by an ocean basin. “O” indicates presence of ophiolite.  Saw teeth indicate suture is a thrust fault

Enumerated_Domain:

Enumerated_Domain_Value: Suture, concealed

Enumerated_Domain_Value_Definition: Boundary between paleo-continents formerly separated by an ocean basin. “O” indicates presence of ophiolite.  Saw teeth indicate suture is a thrust fault

Enumerated_Domain:

Enumerated_Domain_Value: Suture, inferred, queried

Enumerated_Domain_Value_Definition: Boundary between paleo-continents formerly separated by an ocean basin. “O” indicates presence of ophiolite.  Saw teeth indicate suture is a thrust fault

Enumerated_Domain:

Enumerated_Domain_Value: Suture, ophiolite, generalized

Enumerated_Domain_Value_Definition: Boundary between paleo-continents formerly separated by an ocean basin. “O” indicates presence of ophiolite.  Saw teeth indicate suture is a thrust fault

Enumerated_Domain:

Enumerated_Domain_Value: Suture, ophiolite, inferred

Enumerated_Domain_Value_Definition: Boundary between paleo-continents formerly separated by an ocean basin. “O” indicates presence of ophiolite.  Saw teeth indicate suture is a thrust fault

Enumerated_Domain:

Enumerated_Domain_Value: Thrust or reverse fault, generalized

Enumerated_Domain_Value_Definition: Saw teeth face upper plate

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: ES_nonprospective_terranes

Entity_Type_Definition: Nonprospective terrane types.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: line_feature

Attribute_Definition: Nonprospective terrane line feature.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Boundaries of Alpha-Mendeleev and Iceland-Faroe Large Igneous Provinces (LIP), based on aeromagnetic data, concealed

Enumerated_Domain_Value_Definition: The Alpha-Mendeleev LIP is overlain by 0 to >1.5 km of eupelagic or hemipelagic sediment and bedrock samples at one locality consist of alkali basalt 120 to 90 Ma

Enumerated_Domain:

Enumerated_Domain_Value: Boundaries of Alpha-Mendeleev and Iceland-Faroe Large Igneous Provinces (LIP), based on aeromagnetic data, generalized

Enumerated_Domain_Value_Definition: The Alpha-Mendeleev LIP is overlain by 0 to >1.5 km of eupelagic or hemipelagic sediment and bedrock samples at one locality consist of alkali basalt 120 to 90 Ma

Enumerated_Domain:

Enumerated_Domain_Value: Boundaries of Alpha-Mendeleev and Iceland-Faroe Large Igneous Provinces (LIP), based on aeromagnetic data, no line

Enumerated_Domain_Value_Definition: The Alpha-Mendeleev LIP is overlain by 0 to >1.5 km of eupelagic or hemipelagic sediment and bedrock samples at one locality consist of alkali basalt 120 to 90 Ma

Enumerated_Domain:

Enumerated_Domain_Value: Boundary of mid-ocean ridge basalts (MORB) at the seabed, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary of ridge of continental crust in an ocean basin, no line

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Faulted boundary of ridge of continental crust in an ocean basin, concealed

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Faulted boundary of ridge of continental crust in an ocean basin, strike-slip concealed

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Faulted boundary of ridge of continental crust in an ocean basin, strike-slip inferred

Enumerated_Domain_Value_Definition:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: ES_oil_and_gas_fields

Entity_Type_Definition: Oil and gas field locations.

Entity_Type_Definition_Source: IHS Energy

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: FIELD_NAME

Attribute_Definition: Names of oil and gas fields.

Attribute_Definition_Source: IHS Energy

Attribute_Domain_Values:

Attribute:

Attribute_Label: HC_TYPE

Attribute_Definition: Hydrocarbon types of field locations.

Attribute_Definition_Source: IHS Energy

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Gas

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Gas and condensate

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Oil

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Oil/gas

Enumerated_Domain_Value_Definition:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: ES_isopachs

Entity_Type_Definition: Lines indicating thickness of stratigraphic 	interval, in km.

Entity_Type_Definition_Source:

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Attribute:

Attribute_Label: thickness

Attribute_Definition: Thickness of stratigraphic interval, in km.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Attribute:

Attribute_Label: line_type

Attribute_Definition: Isopach line symbol type

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: ES_geophysical_profiles

Entity_Type_Definition: Profiles showing estimated thickness of 	sedimentary strata, in km.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: ES_misc_area_features

Entity_Type_Definition: Diapir and impact structure areas.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: type

Attribute_Definition: Area type.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Diapir

Enumerated_Domain_Value_Definition: Evaporite, locally shale, undifferentiated

Enumerated_Domain:

Enumerated_Domain_Value: Impact structure

Enumerated_Domain_Value_Definition: astrobleme

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: ES_misc_point_features

Entity_Type_Definition: Diapir and Kimberlite pipe locations.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: type

Attribute_Definition: Location type

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Diapir

Enumerated_Domain_Value_Definition: Evaporite, locally shale, undifferentiated. Triangles show locality only

Enumerated_Domain:

Enumerated_Domain_Value: Kimberlite

Enumerated_Domain_Value_Definition: Kimberlite pipe location

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: ES_igneous_rocks

Entity_Type_Definition: Igneous rock areas.

Entity_Type_Definition_Source:

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: type

Attribute_Definition: Igneous rock type.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Igneous intrusions

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Intercalated volcanic rocks

Enumerated_Domain_Value_Definition:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: ES_LIP_interior

Entity_Type_Definition: Areas within Large Igneous Province (LIP) boundaries

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

GREENLAND QUADRANT FEATURE CLASSES:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: GRN_surface_sedimentary_successions

Entity_Type_Definition: Geologic units.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: sedimentary_succession

Attribute_Definition: Geologic unit description.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Basin created by multiple rift and thermo-isostatic (sag) events

Enumerated_Domain_Value_Definition: The extension that created the basin thinned, but did not sever, the underlying continental crust. Typically consist of both siliciclastic and carbonate deposits and in places evaporites

Enumerated_Domain:

Enumerated_Domain_Value: Coastal plain and marine shelf

Enumerated_Domain_Value_Definition: Epicontinental nonmarine and marine shelf deposits of Mesozoic and (or) Cenozoic age, typically no more than 1 or 2 km thick. Overlain by eupelagic deposits on Lomonosov Ridge, Marvin Spur and probably Yermak Plateau

Enumerated_Domain:

Enumerated_Domain_Value: Extensional basin on continental crust along the margin of an adjoining ocean basin. The stretching that created the extensional basin was a precursor to seafloor spreading in the ocean basin

Enumerated_Domain_Value_Definition: Contains mainly siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Foreland basin

Enumerated_Domain_Value_Definition: Basin created by flexural, sedimentary and thrust sheet loading of continental crust in front of a convergence zone. Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Ocean basin

Enumerated_Domain_Value_Definition: Eupelagic, hemipelagic or turbidite deposits overlying mid-ocean ridge basalt (MORB) or ocean-continent transitional (extended continental) crust

Enumerated_Domain:

Enumerated_Domain_Value: Progradational sedimentary succession across predominantly rifted passive margin of an existing ocean basin

Enumerated_Domain_Value_Definition: Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Progradational sedimentary succession across predominantly strike-slip margin of an existing ocean basin

Enumerated_Domain_Value_Definition: Contains siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Stable shelf and platform

Enumerated_Domain_Value_Definition: Commonly contain carbonate as well as clastic deposits

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: GRN_subsurface_sedimentary_successions

Entity_Type_Definition: Geologic units.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain:

Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: sedimentary_succession

Attribute_Definition: Geologic unit description.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

(Numeric values in parentheses contained in the geodatabase feature class, which refer to the angle of the stripes in the fill pattern, have been omitted from the following domain value descriptions.)

Enumerated_Domain:

Enumerated_Domain_Value: Basin created by multiple rift and thermo-isostatic (sag) events, underlying

Enumerated_Domain_Value_Definition: The extension that created the basin thinned, but did not sever, the underlying continental crust. Typically consist of both siliciclastic and carbonate deposits and in places evaporates

Enumerated_Domain:

Enumerated_Domain_Value: Coastal plain and marine shelf, underlying

Enumerated_Domain_Value_Definition: Epicontinental nonmarine and marine shelf deposits of Mesozoic and (or) Cenozoic age, typically no more than 1 or 2 km thick. Overlain by eupelagic deposits on Lomonosov Ridge, Marvin Spur and probably Yermak Plateau

Enumerated_Domain:

Enumerated_Domain_Value: Extensional basin on continental crust along the margin of an adjoining ocean basin. The stretching that created the extensional basin was a precursor to seafloor spreading in the ocean basin

Enumerated_Domain_Value_Definition: Contains mainly siliciclastic deposits

Enumerated_Domain:

Enumerated_Domain_Value: Ocean basin, underlying

Enumerated_Domain_Value_Definition: Eupelagic, hemipelagic or turbidite deposits overlying mid-ocean ridge basalt (MORB) or ocean-continent transitional (extended continental) crust

Enumerated_Domain:

Enumerated_Domain_Value: Stable shelf and platform, underlying

Enumerated_Domain_Value_Definition: Commonly contain carbonate as well as clastic deposits

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: GRN_boundaries_and_structural_axes

Entity_Type_Definition: Geologic line types.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: line_feature

Attribute_Definition: Geologic line feature.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Active sea-floor spreading axis, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Active sea-floor spreading axis, inferred

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Axis of structural arch expressed in configuration of basement, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Axis of structural arch expressed in configuration of basement, inferred

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Axis of structural trough expressed in configuration of basement, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Axis of structural trough expressed in configuration of basement, inferred

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary between geographic subdivisions of a sedimentary succession

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Continent-ocean boundary

Enumerated_Domain_Value_Definition: Approximated from aeromagnetic or seismic data, shown locally

Enumerated_Domain:

Enumerated_Domain_Value: Depositional boundary of a sedimentary succession, concealed

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Depositional boundary of a sedimentary succession, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Depositional boundary of a sedimentary succession, inferred

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Extinct sea-floor spreading axis, generalized

Enumerated_Domain_Value_Definition:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: GRN_faulted_boundary_of_a_sedimentary_succession

Entity_Type_Definition: Faulted boundary of a sedimentary succession or its subdivisions.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: line_feature

Attribute_Definition: Geologic line feature.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Faulted boundary of a sedimentary succession or its subdivisions                            							Enumerated_Domain_Value_Definition: Ornaments as for Faults and Structural Fronts

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: GRN_faults_and_structural_fronts

Entity_Type_Definition: Fault line types.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: name

Attribute_Definition: Name of fault or structural front.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Attribute:

Attribute_Label: fault_type

Attribute_Definition: Fault line and structural front types.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Deformation front, generalized

Enumerated_Domain_Value_Definition: Separates folded from undeformed strata. Ornament faces folded strata

Enumerated_Domain:

Enumerated_Domain_Value: Deformation front, inferred

Enumerated_Domain_Value_Definition: Separates folded from undeformed strata. Ornament faces folded strata

Enumerated_Domain:

Enumerated_Domain_Value: Deformation front, no line

Enumerated_Domain_Value_Definition: Separates folded from undeformed strata. Ornament faces folded strata

Enumerated_Domain:

Enumerated_Domain_Value: Deformational boundary, generalized

Enumerated_Domain_Value_Definition: Separates strata with numerous thrust faults and associated folds from folded strata with relatively few faults. Ornament faces the more intensely deformed strata

Enumerated_Domain:

Enumerated_Domain_Value: Fault, concealed

Enumerated_Domain_Value_Definition: Character not known

Enumerated_Domain:

Enumerated_Domain_Value: Fault, generalized

Enumerated_Domain_Value_Definition: Character not known

Enumerated_Domain:

Enumerated_Domain_Value: Fault, inferred

Enumerated_Domain_Value_Definition: Character not known

Enumerated_Domain:

Enumerated_Domain_Value: Normal fault, concealed

Enumerated_Domain_Value_Definition: Hachures face hanging wall

Enumerated_Domain:

Enumerated_Domain_Value: Normal fault, generalized

Enumerated_Domain_Value_Definition: Hachures face hanging wall

Enumerated_Domain:

Enumerated_Domain_Value: Normal fault, inferred

Enumerated_Domain_Value_Definition: Hachures face hanging wall

Enumerated_Domain:

Enumerated_Domain_Value: Strike-slip fault, concealed

Enumerated_Domain_Value_Definition: Arrows show relative displacement

Enumerated_Domain:

Enumerated_Domain_Value: Strike-slip fault, generalized

Enumerated_Domain_Value_Definition: Arrows show relative displacement

Enumerated_Domain:

Enumerated_Domain_Value: Strike-slip fault, inferred

Enumerated_Domain_Value_Definition: Arrows show relative displacement

Enumerated_Domain:

Enumerated_Domain_Value: Structural front, concealed

Enumerated_Domain_Value_Definition: Separates intensely folded, faulted or metamorphosed strata from strata containing only open folds.  Ornament faces the more intensely deformed terrane, which is considered to lack significant hydrocarbon potential

Enumerated_Domain:

Enumerated_Domain_Value: Structural front, generalized

Enumerated_Domain_Value_Definition: Separates intensely folded, faulted or metamorphosed strata from strata containing only open folds.  Ornament faces the more intensely deformed terrane, which is considered to lack significant hydrocarbon potential

Enumerated_Domain:

Enumerated_Domain_Value: Suture, ophiolite, generalized

Enumerated_Domain_Value_Definition: Boundary between paleo-continents formerly separated by an ocean basin. “O” indicates presence of ophiolite.  Saw teeth indicate suture is a thrust fault

Enumerated_Domain:

Enumerated_Domain_Value: Thrust or reverse fault, generalized

Enumerated_Domain_Value_Definition: Saw teeth face upper plate

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: GRN_nonprospective_terranes

Entity_Type_Definition: Nonprospective terrane types.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: line_feature

Attribute_Definition: Nonprospective terrane line feature.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Boundaries of Alpha-Mendeleev and Iceland-Faroe Large Igneous Provinces (LIP), based on aeromagnetic data, concealed

Enumerated_Domain_Value_Definition: The Alpha-Mendeleev LIP is overlain by 0 to >1.5 km of eupelagic or hemipelagic sediment and bedrock samples at one locality consist of alkali basalt 120 to 90 Ma

Enumerated_Domain:

Enumerated_Domain_Value: Boundaries of Alpha-Mendeleev and Iceland-Faroe Large Igneous Provinces (LIP), based on aeromagnetic data, generalized

Enumerated_Domain_Value_Definition: The Alpha-Mendeleev LIP is overlain by 0 to >1.5 km of eupelagic or hemipelagic sediment and bedrock samples at one locality consist of alkali basalt 120 to 90 Ma

Enumerated_Domain:

Enumerated_Domain_Value: Boundary of mid-ocean ridge basalts (MORB) at the seabed, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary of mid-ocean ridge basalts (MORB) at the seabed, inferred

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary of mid-ocean ridge basalts (MORB) at the seabed, no line

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary of ridge of continental crust in an ocean basin, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary of ridge of continental crust in an ocean basin, no line

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Boundary of the Neogene basaltic volcanic edifice constructed over the Iceland Faroe LIP and surrounding seabed, based on bathymetric data, generalized

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Faulted boundary of ridge of continental crust in an ocean basin, concealed

Enumerated_Domain_Value_Definition:

Enumerated_Domain:

Enumerated_Domain_Value: Faulted boundary of ridge of continental crust in an ocean basin, strike-slip concealed

Enumerated_Domain_Value_Definition:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: GRN_oil_and_gas_fields

Entity_Type_Definition: Oil and gas field locations.

Entity_Type_Definition_Source: IHS Energy

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: FIELD_NAME

Attribute_Definition: Names of oil and gas fields.

Attribute_Definition_Source: IHS Energy

Attribute_Domain_Values:

Attribute:

Attribute_Label: HC_TYPE

Attribute_Definition: Hydrocarbon types of field locations.

Attribute_Definition_Source: IHS Energy

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Gas, condensate and oil

Enumerated_Domain_Value_Definition:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: GRN_isopachs

Entity_Type_Definition: Lines indicating thickness of stratigraphic 	interval, in km.

Entity_Type_Definition_Source:

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Attribute:

Attribute_Label: thickness

Attribute_Definition: Thickness of stratigraphic interval, in km.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Attribute:

Attribute_Label: line_type

Attribute_Definition: Isopach line symbol type

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: GRN_geophysical_profiles

Entity_Type_Definition: Profiles showing estimated thickness of 	sedimentary strata, in km.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: GRN_diapirs

Entity_Type_Definition: Diapir locations.

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: type

Attribute_Definition: Location type

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Diapir

Enumerated_Domain_Value_Definition: Evaporite, locally shale, undifferentiated. Triangles show locality only

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: GRN_igneous_rocks

Entity_Type_Definition: Igneous rock areas.

Entity_Type_Definition_Source:

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: type

Attribute_Definition: Igneous rock type.

Attribute_Definition_Source: Author

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: Intercalated volcanic rocks

Enumerated_Domain_Value_Definition:

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: GRN_LIP_interior

Entity_Type_Definition: Areas within Large Igneous Province (LIP) boundaries

Entity_Type_Definition_Source: Author

Attribute:

Attribute_Label: OBJECTID

Attribute_Definition: Internal feature number.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Sequential unique whole numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE

Attribute_Definition: Feature geometry.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Coordinates defining the features.

Attribute:

Attribute_Label: SHAPE_Length

Attribute_Definition: Length of feature in internal units.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Unrepresentable_Domain: Positive real numbers that are automatically generated.

Attribute:

Attribute_Label: SHAPE_Area

Attribute_Definition: Area of feature in internal units squared.

Attribute_Definition_Source: ESRI

Attribute_Domain_Values:

Resource_Description: Downloadable Data

Distribution_Liability:

Any use of trade, product, or firm names is for descriptive purposes only and does not imply endorsement by the U.S. Government. Although this publication has been subjected to rigorous review and is substantially complete, the USGS reserves the right to revise the data pursuant to further analysis and review. Furthermore, it is released on condition that neither the USGS nor the United States Government may be held liable for any damages resulting from its authorized or unauthorized use.

Metadata_Reference_Information:

Metadata_Date: 2010

Contact_Information:

Contact_Person_Primary:

Contact_Person: Arthur Grantz (Consulting Geologist)

Contact_Organization: U.S.G.S World Energy Project

Contact_Address:

Address_Type: mailing and physical address

Address: 930 Van Auken Circle

City: Palo Alto

State_or_Province: CA

Postal_Code: 94303

Country: USA

Metadata_Standard_Name: FGDC Content Standards for Digital Geospatial Metadata

Metadata_Standard_Version: FGDC-STD-001-1998

Metadata_Time_Convention: local time

Metadata_Extensions:

Online_Linkage: <http://www.esri.com/metadata/esriprof80.html>

Profile_Name: ESRI Metadata Profile